Rudolf,

(another late night)... but this time I'm getting somewhere!

BeTVOutVideoConsumer now displays live video!

There are still a lot of bugs/problems that I need to fix, mostly due

to my limited/faulty understanding of the media kit and media nodes.

But for now, the video output is working, in 640x480 Res (PAL

'Centered', NTSC 'Centered', and NTSC 'Overscan'), and in color spaces

RGB15, RGB16, and RGB32.

The code currently sets up three 640x480 buffers in the graphics card's

framebuffer, and then passes these to the caputer card (so called

'buffer producer') In this way, the capture card actually 'writes' the

captured frames *directly* into video memory, across the PCI bus. My

CPU load shows near 2%!!!

For some odd reason, If I make more than three buffers, and send them

to the Bt848 capture node, the node never writes any data into the

buffers. Weird!

I have not yet incorporated the field signal or any retrace_sync, so as

you can imagine, things can get pretty 'shaky' looking. Actually the

'flicker filter' masks this problem quite well, I guess by

interpolating over both odd an even scanlines... Perhaps I will work

more on this tomorrow.

But as a first 'proof of concept', I think this is good!!

Feel free to test it out a bit and tell me what you think. It is pretty

stable, no crashes here (except I can't get B_GRAY8 color to work)... I

have no idea how broken CH support might be! (it is also still compiled

for dano)

Gotta go to sleep now,

-Kevin H. Patterson