This is the Packet Loader (PC side).

The protocol it's almost like old BJL loader.

Up-Loading :

Note: Allways sending longs (32bit), and in big endian.

-1) Send packet loader using BJL protocol, (magic, start, length+4, data, checksum)

The packet loader is into the executable.

0) Send your file using Packet Loader protocol

Packet Loader protocol:

1) Send magic $22071970.

2) Send start address (which is also used as destination address)

3) Send length in bytes (be sure its a multiply of 4 !!)

4) Send packet (as longs !), packet size it's hardcoded to 4096 bytes (1024 longs)

5) Send checksum long:

check=-Sum of longs

6) Read one bit

7) If TRUE continue with next packet

8) If FALSE repeat same packet

9) After all packets are read the program will automatically start

Sending a long:

1) Wait for BUSY to be low => inactive

2) Send LSB

3) Set /STROBE to low => active

4) Wait for BUSY high => active

5) Set /STROBE to high

6) continue for the next three bytes

Receiving a bit:

I coded a routine to receive (PC side) and send (Jaguar side) a byte in a serial way using BUSY bit.

It was coded using 68000, and everything was ok, but when I coded it using GPU i had some troubles.

Any way this is the protocol, (Jaguar side).

1) Set BUSY low	=> tell we are ready to send

2) Wait for /STROBE to low	=> PC is ready to send

3) Put BUSY equal to MSB (bit 7)

4) Wait for /STROBE to high	=> PC has readed the bit

5) Loop to 2 until all bits are sended (remember 8 bits)